To Send“homegrown” SID data to Stanford University

http://sid.stanford.edu/database-browser/

- File format:

Your data must be in the standard SID data file format, which is written in ascii text (readable in Excel in addition to the SID tools).  The file starts with a series of “# keyword=value” pairs, which may be listed in any order.  Then the data follow. Usually one data file represents one 24-hour day, though other options, as well as partial data files, are acceptable.   The keywords are explained in detail in Appendix A.   The data are explained in Appendix B.  Here’s an example data file:

# Site = ATL_OU

# MonitorID = 0297

# Contact = Lee Smith lee@mailserver.com

# Latitude = -84.33

# Longitude = 33.87

# UTC_StartTime = 2013-08-14 00:00:00

# TimeZone = EDT

# UTC_Offset = 20

# StationID = JJI

# Frequency = 22200

# SampleRate=  5

2013-08-14 00:00:00, 17662035.3297

2013-08-14 00:00:05, 17662035.3297

2013-08-14 00:00:10, 17662035.3297

2013-08-14 00:00:15, 17662035.3297

2013-08-14 00:00:20, 17944484.6378

2013-08-14 00:00:25, 18226933.9459

2013-08-14 00:00:30, 18509383.2539

-------   etc.

2013-08-14 23:59:20, 13041515.1864

2013-08-14 23:59:25, 11411325.7881

2013-08-14 23:59:30, 9781136.38981

2013-08-14 23:59:35, 8150946.99151

2013-08-14 23:59:40, 6520757.59321

2013-08-14 23:59:45, 4890568.19491

2013-08-14 23:59:50, 3260378.7966

2013-08-14 23:59:55, 1630189.3983

- File naming conventions:

The SID data processing software picks up information from SID data file names, so the names must be in a certain format.  The format of the file names is:

site_stationid_yyyy-mm-dd.csv

The site should match the “Site” value in your data file.  The stationid should match the “StationID” value in your data file.  And the yyyy-mm-ddshould match either the first part or all of theUTC_StartTime value in your file.

Examples are:

SARA_JJI_2013-04-15.csv

Romania-2_NAA_2013-01-30.csv

ATL_OU_NWC_2013-08-14.csv

- How to ftp:

You may send your SID data to a specific Stanford machine set up to accept anonymous ftp (file transfer protocol):

sid-ftp.stanford.edu

If you are not familiar with ftp, you should read up on it before attempting to transfer files.

Once you understand how to use ftp, you will end up doing something like this to transfer a single data file:

ftp sid-ftp.stanford.edu

[use “anonymous” as the user login name, and your email address as the password]

putyourfilename /incoming/SuperSID/yourfilename

exit

The ftp command will usually indicate if the attempt has succeeded or failed.  If it failed, there is probably a firewall problem with your own system.  You will need to track that down, or talk with a System Administrator.

If you have succeeded in sending a single file, then you will probably want to set up a script to automatically send files every day or so.

- Checking your data files in the SID database:

SID data files are picked up from the ftp machine and processed approximately once every hour.   To see your files in the SID databrowser, go to:

http://sid.stanford.edu/database-browser/

If your ftp has succeeded and enough time has passed to have your files processed, you can check your data by clicking on the “View Data by Monitor” hotlink.   Type your site name or your MonitorID into the “Search” box and you should see your monitor and the transmitters (stations) for which you have sent data.  If your site or monitor does not show up, your data have not (yet?) been included in the database.  Contact someone at Stanford for help.

Appendix A – Keywords

The keyword=value pairs must be the first thing in your data file, each preceded by “#”.  The pairs may be listed in any order.   The only critical keywords are Site, MonitorID, UTC_StartTime, and StationID.  If any of these are missing, the files cannot be processed.   Contact, Latitude, and Longitude are useful, but not required.  The others are not currently being used, though they may be used in the future.  SID processing code will remove spaces from all keywords and convert spaces in the value portion to underscores (as required by our database system).

| Keyword | Example Values | Comments |
| --- | --- | --- |
| Site | WSO orSARA_SS or ITALY-2 | A short name that you chose, or were assigned, to identify your site (best to avoid spaces) |
| MonitorID | 174 or S-0139 or MyInstrument1 | The serial number labeled on your SuperSIDinstrument. If none, then make up an identifier for your instrument. These must be unique in the SID community! |
| Contact | Joe Smith joe@someplace.com | It is very helpful to have a way to contact the provider of data files. Please include your name and email address! |
| Latitude | 37.41 | Your site’s latitude in decimal format |
| Longitude | -122.17 | Your site’s longitude in decimal format |
| UTC_StartTime | 2013-08-14 00:00:00 | The initial timestamp in your data file. |
| TimeZone | PST | Your time zone. See http://www.worldtimezone.com/wtz-names/timezonenames.html |
| UTC_offset | -07:00 | The number of hours you are different from Universal Time (See http://www.worldtimezone.com/). |
| StationID | JJI | The 3-letter code name of the transmitter which this file is monitoring |
| Frequency | 22200 | The frequency of the transmitter this file is monitoring. See http://sidstation.loudet.org/stations-list-en.xhtml |
| SampleRate | 5 | The SuperSIDs sample every 5 seconds (i.e. signal strengths are measured and logged into a data file once every 5 seconds). Changing this value will not change your sample rate! If you have a home-grown SID, indicate your sample rate here. (Currently this keyword is ignored.) |
| Comment or other word you define |  | You may also define your own keywords. They will be ignored by the SID data processing software but left in your data files. |

Appendix B – SID Data

SID data are kept as timestamp, value pairs, with one pair per line, written in ascii, and preceded by a set of # keyword=value pseudo-data.   Here’s an example data line:

2013-08-14 23:59:25, 11411325.7881

The date must be given in the standard yyyy-mm-ddhh:mm:ss format.

The data value is a decimal number.  The range of these values is dependent upon the particular instrument being used.  The range could be between 0.0 and 1.0, or between 13041515.1864 and 6520757.59321, or any other reasonable set of numbers.  (The American convention is used – i.e. a period “.” separates the integer part of the number from the decimal value.)  The numbers are (somewhat) normalized before being graphed by the SID databrowser.    Because each instrument is different, the actual numbers in the various data files cannot be compared directly with each other, only relatively.

A comma “,” must separate the timestamp from the data value.  You may have as many data lines in the file as you want.  However, most files do not exceed a 24-hour period.