Tracking Noise Interference
If you seem to be getting a lot of noise interference, here are some steps to take to possibly find the source if it is local.
Start SuperSID (SSID)
The Antenna and Coax- Make sure that the wires are not old and all the connections are clean, no oxides on surfaces. The presence of oxides makes for poor conduction and in some cases it results in rectification of signals which can then lead to a host of other problems.
Clean any oxide and repair any bad connections.

Has your SSID spectrum improved?

If Yes, continue to section C.  If No, let’s look at your computer to see if it is the culprit.

Move the antenna next to the computer, allow SSID at least 20 seconds to respond.  Do you see any change good or bad? If the signal is worse, move the antenna as far away from the computer as possible, any change?

Section C- Turn off all electrical equipment in the building (if possible).  Is there an improvement to the signal with all electrical items turned off?  If yes, continue with the following steps.

If no, it may be a noise source out side the building.

Turn On each item and them look at the SuperSID spectrum, give SuperSID time (at least 20 seconds) to refresh and see if there is any change
